Series

BWC-M	Heating Cable for Concrete on Mat

Features

Voltage	-	240/208V, 347V, 1-phase.

Construction	-	Twin conductor heating cable attached to a plastic mat

with negligible magnetic field.

Watt density	-	11W/sq. ft. (120W/sq. m), factory installed on mat at 6’’ (15 cm) spacing.

Dimension	-	24’’ (0.6 m) wide mat available in several lengths.

Cold lead length	-	8’ 2’’ (2.5 m) cold lead included.

Control	-	Two types of control method possible (see instruction manual for details):

• Surface heating control with electronic thermostat in floor mode (F) and temperature sensor.

•	Ambient heating control with electronic thermostat in ambient mode with floor limit (A or AF) and temperature sensor.

Note: A ground fault circuit interrupter (GFCI) must be used with this heating

device unless exempted by the applicable national and/or local electrical

code for the area of installation.

Included materials	-	15' (4.6 m) temperature sensor.

-	Optional 50’ (15 m) cold lead available upon request.

-	Plastic floor fasteners (KIT-WC-CLP).

-	Measurement table label (to be placed in for electrical panel).

Installation	-	Never cut or shorten the heating cable.

-	For indoor applications only, residential or commercial.

-	Installs under a 4" to 6" (10 cm to 15 cm) concrete slab or under a 1.5’’ to 4’’ concrete topping (4 cm to 10 cm).

Note: It’s highly recommended to insulate the concrete slab in order to avoid

heat loss from below (see instruction manual for all installation details).

-	Compatible with most floor coverings (check with the dealer or manufacturer).

-	Installation with or without metallic structure for reinforced concrete.

Warranty	-	20-year warranty on the heating cable.

Application 	-	Basement, garage, bathroom, kitchen, family room, workshop, pool, shower, entrance way, hospital, hotel, factory, business, restaurant, sunroom, greenhouse, buildings used for housing animals.

Models

| Watts | Product # |  |  |  | Covered surface |  | Length |  | Weight |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 240/208V | Qty | 347V | Qty | sq. ft. | sq. m | ft. in. | m | lb | kg |
| 150 | BWC-M0150 |  | - | - | 14.0 | 1.3 | 6’ 5" | 1.9 | 3.0 | 1.4 |
| 200 | BWC-M0200 |  | - | - | 19.0 | 1.7 | 8' 4" | 2.5 | 3.7 | 1.7 |
| 300 | BWC-M0300 |  | - | - | 28.0 | 2.6 | 12’ 6" | 3.8 | 4.0 | 1.8 |
| 400 | BWC-M0400 |  | - | - | 38.0 | 3.5 | 16’ 8" | 5.1 | 4.5 | 2.0 |
| 500 | BWC-M0500 |  | - | - | 46.5 | 4.3 | 20’ 10" | 6.4 | 5.0 | 2.3 |
| 600 | BWC-M0600 |  | - | - | 56.0 | 5.2 | 25’ | 7.6 | 6.0 | 2.7 |
| 700 | BWC-M0700 |  | - | - | 65.5 | 6.1 | 29’ 2" | 8.9 | 7.0 | 3.1 |
| 850 | BWC-M0850 |  | - | - | 80.0 | 7.4 | 35’ 5" | 10.8 | 8.0 | 3.6 |
| 950 | BWC-M0950 |  | - | - | 89.0 | 8.3 | 39’ 7" | 12.1 | 9.0 | 4.0 |
| 1100 | BWC-M1100 |  | - | - | 103.0 | 9.6 | 45’ 10" | 14.0 | 10.0 | 4.5 |
| 1200 | BWC-M1200 |  | - | - | 113.0 | 10.5 | 50' | 15.2 | 11.0 | 5.0 |
| 1300 | BWC-M1300 |  | - | - | 121.5 | 11.3 | 54’ 2" | 16.5 | 12.0 | 5.4 |
| 1400 | BWC-M1400 |  | - | - | 130.5 | 12.1 | 58’ 4" | 17.8 | 13.0 | 6.0 |
| 1500 | BWC-M1500 |  | - | - | 140.5 | 13.1 | 62’ 6" | 19.1 | 14.0 | 6.4 |
| 1600 | BWC-M1600 |  | - | - | 149.5 | 13.9 | 66’ 8" | 20.3 | 17.0 | 7.8 |
| 1700 | BWC-M1700 |  | - | - | 159.0 | 14.8 | 70’ 10" | 21.6 | 16.0 | 7.2 |
| 1850 | BWC-M1850 |  | - | - | 172.5 | 16.0 | 77’ 1" | 23.5 | 17.0 | 7.8 |
| 2000 | BWC-M2000 |  | BWC-M2007 |  | 187.5 | 17.4 | 83’ 4" | 25.4 | 18.0 | 8.1 |
| 2200 | BWC-M2200 |  | - |  | 206.0 | 19.1 | 91’ 6" | 27.9 | 21.0 | 9.5 |
| 2400 | BWC-M2400 |  | BWC-M2407 |  | 225.0 | 20.9 | 100’ | 30.5 | 23.0 | 10.4 |
| 2550 | BWC-M2550 |  | - |  | 239.0 | 22.2 | 106’ 6" | 32.5 | 25.0 | 11.3 |
| 2700 | BWC-M2700 |  | BWC-M2707 |  | 253.0 | 23.5 | 112’ 6" | 34.3 | 28.0 | 12.7 |
| 2850 | BWC-M2850 |  | - |  | 267.0 | 24.8 | 119’ | 36.3 | 30.0 | 13.6 |
| 3000 | BWC-M3000 |  | BWC-M3007 |  | 281.0 | 26.1 | 125’ | 38.1 | 32.0 | 14.5 |
| 3200 | BWC-M3200 |  | - |  | 300.0 | 27.9 | 133’ 6" | 40.7 | 34.0 | 15.4 |
| 3400 | BWC-M3400 |  | BWC-M3407 |  | 318.5 | 29.6 | 141’ 8" | 43.2 | 36.0 | 16.3 |
| 3600 | BWC-M3600 |  | - |  | 336.0 | 31.2 | 150’ | 45.7 | 38.0 | 17.2 |
| 3700 | - | - | BWC-M3707 |  | 346.5 | 32.2 | 154’ 2 | 47.0 | 39.0 | 17.7 |
| 4000 | - | - | BWC-M4007 |  | 375.0 | 34.8 | 166’ 8" | 50.8 | 42.0 | 19.0 |

Does not represent the room surface but rather the area covered by the cable mat including 3” (7.5 cm) spacing between the mat strips but excluding fixed elements to be bypassed and any other required clearances.

208V = 75% of wattage at 240V.

Options

| Product # Kit | Product # Factory installed* | Description | Qty |
| --- | --- | --- | --- |
| OTM-CC1 | - | CableCheck – Electrical indicator |  |
| KIT-WC-CLP | - | Bag of 50 plastic floor fasteners for heating cable on mat |  |
| KIT-SP2 | - | Repair kit |  |
| KIT-CBL-SN | - | 15ft. (4.6 m) floor sensor |  |
| - | - 503 | Optional 50’ (15 m) cold lead |  |
| WC-1SSR-XXX2 | - | GFCI control panel with 1 pair of SSR relays and 24V control transformer |  |
| WC-2SSR-XXX2 | - | GFCI control panel with 2 pairs of SSR relays and 24V control transformer |  |
| WC-3SSR-XXX2 | - | GFCI control panel with 3 pairs of SSR relays and 24V control transformer |  |
| WC-4SSR-XXX2 | - | GFCI control panel with 4 pairs of SSR relays and 24V control transformer |  |
| WC-5SSR-XXX2 | - | GFCI control panel with 5 pairs of SSR relays and 24V control transformer |  |
| WC-6SSR-XXX2 | - | GFCI control panel with 6 pairs of SSR relays and 24V control transformer |  |
| WC-12SSR-XXX2 | - | GFCI control panel with 12 pairs of SSR relays and 24V control transformer |  |

* For factory installed options, add the option number to the product number.

1 With any BWC order, the accessory OTM-CC can be added free of charge upon costumer request. 
2 XXX = 208, 240, 347 Volts.

3 Made to order only. Allow additional 9 to 12 weeks lead time.

### Others

| Type | Quantity | Model et description Complementary informations | Volts | Phase | Watts | Length mm/in. | Informations Color/installation |
| --- | --- | --- | --- | --- | --- | --- | --- |

Specifications may change without prior notice

| Project: | Approval Information |
| --- | --- |
| Consultant Engineer: |  |
| Eectrician Contractor: |  |
| Distributor: |  |
| Date: |  |
| Contact in Ouellet: |  |
| Comments: |  |